
	Δελτίο της Ελληνικής Γεωλογικής Εταιρίας, τομ. XLVII , 2013

Πρακτικά 13ου Διεθνούς Συνεδρίου, Χανιά, Σεπτ. 2013
	of the Geological Society of Greece, vol. XLVII 2013

Proceedings of the 13th International Congress, Chania, Sept. 2013Bulletin 


DEFINITION OF INFERRED FAULTS USING 3D GEOLOGICAL MODELING TECHNIQUES: A CASE STUDY IN TYMPAKI BASIN IN CRETE, GREECE
Panagopoulos G.1, Giannakakos E.1, Manoutsoglou E.1, Steiakakis E.2, Soupios P.3 & Vafidis A.4
1Technical University of Crete, Dept. of Mineral Resources Engineering, Research Unit of Geology, Chania, Greece, gpanagopoulos@isc.tuc.gr, emanout@mred.tuc.gr.

2Technical University of Crete, Dept. of Mineral Resources Engineering, Laboratory of Applied Geology, Chania, Greece, stiakaki@mred.tuc.gr.

3Department of Natural Resources & Environment, Technological Educational Institute of Crete, soupios@chania.teicrete.gr
4Technical University of Crete, Dept. of Mineral Resources Engineering, Laboratory of Applied Geophysics, Chania, Greece, vafidis@mred.tuc.gr.
Abstract 

The purpose of this paper is to recognize inferred faults in Tympaki basin (Crete island) which cannot be observed on the surface, as the area is covered by recent deposits. The identification of the faults is crucial for the study of the sea-water intrusion in Tympaki basin that has been observed during the last years. 
3D geological modelling techniques were used for reaching the aforementioned goal. The data used include the digital elevation model (DEM) of the area, the boundary surface between geologic formations and the litho - stratigraphic logs of thirty nine (39) wells. Initially a 3D stratigraphic model was set-up depicting the spatial extension of Plio-Quaternary and Neogene deposits. Next, with emphasis on the litho - stratigraphic logs of the wells, the boundary between the geological formations was defined in a 3D space. 

The regional unconformity between the Plio-Quaternary and Neogene formations is possibly the result of tectonic activity of inferred faults. The traces of the inferred faults were drawn and they were compared with the faults suggested by previous studies.    
Key words: geological structure, 3D geological modelling technique, inferred faults
Περίληψη
Σκοπός της παρούσας εργασίας είναι ο καθορισμός πιθανών ρηγμάτων στη λεκάνη του Τυμπακίου τα οποία δεν είναι δυνατόν να παρατηρηθούν στην επιφάνεια εξαιτίας της κάλυψης της περιοχής από πρόσφατες αποθέσεις. Ο καθορισμός ρηγμάτων αποτελεί σημαντική στοιχείο στη μελέτη του προβλήματος υφαλμύρινσης που έχει παρατηρηθεί εκτεταμένα  στη λεκάνη του Τυμπακίου τα τελευταία χρόνια. Η χρήση μεθόδων τρισδιάστατης γεωλογικής μοντελοποίησης αποτελεί τον τρόπο προσέγγισης αυτού του στόχου και απαιτεί τη συλλογή του συνόλου των διαθέσιμων χωρικών στοιχείων σε μια ενιαία βάση δεδομένων. Τα δεδομένα που εισήχθησαν αφορούσαν το ψηφιακό υψομετρικό μοντέλο (DEM), τα επιφανειακά όρια των γεωλογικών σχηματισμών και τις λιθοστρωματικές στήλες από τριάντα εννέα (39) γεωτρήσεις. Αρχικά, κατασκευάστηκε το τρισδιάστατο λιθοστρωματογραφικό μοντέλο που παρουσιάζει τη χωρική κατανομή των Πλειο-Τεταρτογενών και Νεογενών αποθέσεων. Ακολούθως, με βάση τις λιθοστρωματογραφικές κολώνες των γεωτρήσεων ορίστηκαν τα γεωλογικά όρια μεταξύ των σχηματισμών στον τρισδιάστατο χώρο. Η ζώνη μετάβασης από τη μια ενότητα στην άλλη τοπικά εμφανίζεται σε διαφορετικό υψόμετρο. Τέτοιες απότομες αλλαγές θα μπορούσαν να αποδοθούν στη δράση πιθανών ρηγμάτων. Τελικά, μετά τη χωροθέτηση των πιθανών ρηγμάτων όπως προέκυψαν από τη συγκεκριμένη μεθοδολογία, γίνεται σύγκριση με τα αποτελέσματα προηγούμενων ερευνών.
Λέξεις κλειδιά: γεωλογική δομή,  πιθανά ρήγματα,  τριδιάστατη γεωλογική μοντελοποίηση
1. Introduction 
Crete is a structurally complex area that has been strongly affected by the Alpine orogenic processes in Eastern Mediterranean, due to plate convergence of Eurasian and African plates and the subduction of the Tethyan oceanic crust (Bonneau 1984; Lister et al., 1984; Mountrakis et al., 2006; Ring et al., 2010). Nappe stacking and compression alternate with extension, nappe collapse and exhumation of deep crustal rocks (Bonneau, 1984; Seidel et al., 1982; Kilias et al., 1994, 2002; Jolivet et al., 1996). 

The island of Crete is situated in a fore-arc position above the active northward-directed subduction zone of the African plate beneath the Aegean plate (Petereck and Schwarze 2004). Present-day convergence rates between Libya and Crete reach 30 mm/yr in a SSW (Oral et al., 1995, Reilinger et al., 1997, McClusky et al., 2000). Given that the movement of Africa towards northward direction is approximately 10 mm/a, the overall plate convergence rate is approximately 40–45 mm/a. This convergence results to the NE-SW strong convergence compression (perpendicular to the African–Aegean plate boundaries) and to moderate extension to an arc-parallel E-W extension field (Petereck and Schwarze, 2004 and references therein). 

The highly mountainous landscape in Crete and the deposits of Middle Miocene to early Late Pliocene age, which uplifted up to several hundreds of meters above the present sea-level, suggest a quite recent and rapid uplift. Along the Hellenic arc and south of Crete, the oblique convergence of the two plates explains the existence of fore-arc troughs controlled by strike-slip faults (Peterek and Schwarze, 2004 and references therein).

A large number of surface faults, such as Klima fault, Kardiotissa fault and others, indicate an intense tectonic activity in the region. The present-day surface deformation of south-central Crete is still a matter of debate. Mostly, a multi-directional extension (Duermeijer et al., 1998) has been proposed, based on fault slip data which are however hardly interpreted. Nevertheless, a dominating arc-parallel extension during Late Pliocene to Holocene (references?)  time is also postulated due to young surface faulting. The existence of sinistral wrenching is evaluated as the main cause of evolution of the isolated Pliocene and younger basins in south-central Crete (Peterek and Schwarze, 2004 and references therein). 

Moreover, recent studies (Chatzaras et al 2006; Tortorici et al., 2010; Kokinou et al., 2012; Klein et al., 2013) cast doubts on the duration of extension as a constantly major tectonic regime, suggesting that compression has played a crucial role beyond Middle Miocene. All these different views support the aspect that highly active tectonic setting of Crete affects the integrity and continuity of the stratigraphical units. This led to several fault-bounded blocks which bring in contact different stratigraphic units.

Several projects (FAO, 1969, 1972 & MEDIS, 2005) have been conducted in Tympaki area during the previous years. The first study performed by FAO (1969). The hydrogeological setting of the whole Messara plain (including Tympaki area) was defined, based on numerous drilling and geophysical data. More recently, in 2005 the MEDIS research project focuses for the first time on the seawater-intrusion phenomenon that had been observed in the Tympaki costal area. 

The scope of the present paper is to contribute to the clarification of the tectono-stratigraphical structure of Tympaki basin, especially in the area which is covered by recent deposits, where no surface data exist. Such information will contribute to defining the hydrogeological setting of the basin.  

2.  Geological setting 
Crete consists of a pile of nappes that contain rock units from various geotectonic zones. These rocks belong both to External Hellenides, namely Phyllite (P-Q), Tripolis, Pindos Nappes and to Internal Hellenides, namely Arvi, Miamou, Vatos, Asterrousia Units and ophiolites, all stacked southward on the parautochthonous Permian-Tertiary Plattenkalk Group (Bonneau, 1984). The uplift and exhumation of the basement was accompanied by structural disintegration of the hanging wall, leading to formation of sedimentary basins on top of the nappe pile from Lower to Middle Miocene. Neogene sediments of Crete were subdivided by Meulenkamp (1979) into six stratigraphical groups, namely Prina, Tefelion, Vrysses, Hellenikon, Foinikia and Agia Galini Groups and the undifferentiated Pleistocene. The sedimentary characteristics of Neogene deposits reflect the tectonic regime of the Hellenic fore-arc at the time of their deposition.

The broader area of Tympaki is the westernmost part of Messara basin that is surrounded by Idi Mountains in northwest, Dikti Mountains in the northeastern side and the Asteroussia mountains in the south. Sedimentation in Messara basin started during the Middle Miocene and it resulted in deposition of sequences of conglomerates, cobbles, sands, marls and clays with abrupted lateral and vertical lithological changes.

Tympaki basin is separated from Messara basin by a tectonic horst that is named “Faistos horst” and it is oriented NNE-SSW (Figure 1). Faistos horst is filled with Neogene deposits, which are regarded as aquitard and separates hydrogeologically the Tympaki basin from the eastern part of Messara basin. There is only an approximately 2km - wide passage through the horst, on which Geropotamos river flows towards the west. 
Tympaki basin exhibits a pronouncedly differentiated relief, with an almost flat south-western domain proximal to the coast, and a north eastern hilly domain proximal to Idi Mountains. The hilly landscape is because of the action of Klima fault to the north and Kardiotissa fault to the east.
The majority of Tympaki basin, especially the south western part, is covered by recent fluvio-lacustrine and alluvial deposits, while the north western domain is covered by fluvial deposits and alluvial fans (Figure 1). According to previous projects (FAO, 1972; Paritsis, 2005), the borders of the basin are defined by the tectonic contact of the aforementioned deposits with normal faults with the Neogene formations, as it is shown in the geological map of the area (Vidakis et al., 1994 and Bonneau, 1985). Apart from the fault boundaries of Tympaki basin, the same studies suggest the existence of additional normal faults that configure crucially the geometry of Tympaki basin. More specifically, according to the results of MEDIS program (Paritsis, 2005), there are also three additional inferred faults in the area (F1, F2, F3 depicted in Figure 2) which are not shown in the geological map of IGME (Vidakis et al., 1994 and Bonneau, 1985). The claim regarding the existence of these normal faults is based on the litho - stratigraphic logs of deep wells and on hydrogeological data retrieved from FAO project (FAO, 1972). These faults comprise a strike in a mean direction from NNW to SSE (Figure 2). In addition, according to a morphotectonic study in the same basin, conducted by Peterek & Schwarze (2004), similar faults exist. 
[image: image1.jpg]



Figure 1: Geological map of Tympaki area (after Vidakis et al., 1994 and Bonneau, 1985, modified). Dashed line shows the limits of Faistos horst. 
[image: image2.png]



Figure 2: Geological map of the Tymbaki basin (Paritsis, 2005). Different colours refer to different lithologies based on geological map of IGME (Vidakis et al., 1994 and Bonneau, 1985). The black dash-dot line delimits the basin boundary. F1, F2, F3 are the postulated normal faults dissecting the aquifer.

3. Methodology

3.1. General

In order to construct the 3D geological model of Tympaki basin, a GIS geological database (ArcGIS) with all the available data was created. Although data integration and cross-correlation of geology can be achieved in two dimensions by the GIS package, it cannot build complex surface or volumes such as geological folding and faulted structures (Kaufmann & Martin, 2008) in three dimensions. The limited capabilities in handling real 3D data with GIS packages, can be overcome using a specialized 3D geomodeling software. 

Based on the above, the GIS database of Tympaki area was imported in the geomodeling software (RockWorks15, RockWare, Inc) and it was integrated by the 3D modelling process. The combined use of both types of software is desirable because it generates more complete and reliable models (Fallara et al., 2006).

The imported data comprises the digital elevation model (DEM) of the area, the surface-geological boundaries and thirty nine (39) well logs. DEM was produced from elevation data (National Cadastre & Mapping Agency S.A.) acquired from a regular grid with cell dimensions of 5m x 5m. In addition, the surface geological data comprise the digitized formation boundaries and the faults lines. Moreover, based on detailed lithological descriptions of the well logs (FAO, 1969; Municipality of Faistos), the geological formations were grouped into 23 lithological units. They were further summarized into two startigraphic units, namely Plio-Quaternary Unit and Neogene Unit (Figure 3). The former consists of clay to pebble deposits and the latter consists mainly of marls and marly limestones. It was assumed that marl formation with thickness over 10m could be regarded as a part of the Neogene. 
	Lithological units
	Stratigraphic units

	[image: image3.png]clay
sity cay
sandy clay
gravelly clay
sond

15
ciyey sand =
Eyr =
grovely sand o=z
£
ciayey sit

o=

sandy sitt
gravel
clayey gravel
sity gravel

=
[@=9)
oo
=
==
sandy gravel [
(]
[ ]
(@
[z
oxoxg

marly gravel
marl

clayey marl
sandy marl
gravelly marl
sandstone
conglomerate




[image: image4.png]clay
sity cay
sandy clay
gravely clay
sond

ayey sand
ity sand
gravely sand
it
dayeysit
gravely it
sondy st
gravel
cayey gravel
ity grovel
sandy grave
marly gravel
mart

clyey mar
sandy mar
gravelly mar
sandstone

conglomerate




	[image: image5.png]Plio_Quatemary.
Neogene






Figure 3: The litho stratigraphic units loaded in the geomodeling software to define the structural model. 

3.2. Application of 3D modelling for the definition of covered structural model.

In sites covered by recent deposits the recognition of tectonic events is served only by morphotectonic methods, especially by the DEMs. However, there are several cases that none morphotectonic feature can be observed, like an area with flat topography. Tympaki basin is such a case, with small changes in surface elevation mainly in the southwestern part of the basin. The only way to approach the covered geological structure is to gather all the available data and to identify sound changes in the lateral continuity of the strata with reference to the borehole logs. 

In the present study thirty nine (39) borehole logs were used to construct the 3D lithostratigraphic model of the area. The maximum depth of the wells reaches at about 350-400m mainly for the wells that have been drilled in the hilly part of the area. The spatial data manipulation was made easier by using the 3D modelling software. Based on the lithological descriptions of the borehole logs, the strata were subdivided into two main lithostratigraphic units, namely Plio-Quaternary and Neogene Units (Fig. 3). Twenty five (25) of the boreholes have drilled the Neogene formation near the basin borders, while none of the boreholes have reached the underlying pre-Neogene basement. 

The digital 3D model was built by using all the existing data sets and the spatial distribution of the geological bodies was defined so as to be able to locate the upper and lower surface of each geological unit in any part of the basin. Especially, the definition of the surface that bounds the Plio-Quaternary and Neogene Units is crucial because any abrupt alteration in elevation could be able to supply very useful information for the definition of a fault that cannot be observed in the surface (Figure 4). The design of a contour-map for this surface is a quick and easy task, when using 3D geological modelling techniques, and it is really useful for the detection of these abrupt changes in a basin-wide scale, constituting a support tool in the design of a fault.
[image: image6.jpg]Areas where abrupt change in the depth of the boundary surface occurs





Figure 4: The concept for identifying areas in the surface where the existence of a fault it is probable. The abrupt change of the boundary surface in depth serves as a guideline.

The surface was defined by interpolating the elevation of the lithostratigraphic contact observed in each borehole, with the remaining area between the boreholes. In this case the boreholes are regarded as reference points. 

In the present study the used interpolation algorithm is triangulation. The accuracy of the resulting surfaces can be evaluated by cross-validating the model. During this process each of the observed values is regarded as unknown and the algorithm runs every time to calculate a value for it. Then, predicted and observed values are correlated, in order to calculate the correlation coefficient and to conclude regarding the reliability of the results. 

Data from 25 out of 39 boreholes were used for cross-validation process, because these wells have reached up to the stratigraphical contact between Plio-Quaternary and the Neogene Units in the area (inside the basin borders). 

4. Results
The boreholes data have been checked through geological sections and fence diagrams and finally led up a geological 3D model of the studied area (Figure 5). The surface that bounds the Neogene (lower) and Plio-Quaternary (upper) formation was defined reasonably well. The cross-validation showed that only 3 of the 25 predicted values were slightly to strongly different in relation to the real values, while the correlation coefficient estimated equal to 0.9959 (Figure 5). 

This surface was used as the leading surface to locate areas of any abrupt alteration in the elevation of the stratigraphic contact between Plio-Quaternary and the Neogene Units. Such abrupt alterations are interpreted in the present paper as the result of tectonic activity and for the further identification of possible faults. In order to locate easily the abrupt changes, an isodepth map showing the surface that separates the Plio-Quaternary and Neogene deposits was produced (Figure 6). The construction of the isodepth map can give a general perspective of the changes in the absolute elevation of the upper surface of Neogene formation that occur in the basin (Figure 6). Where abrupt changes occur, the existence of possible faults can be assumed. Based on this concept (Figure 3) several faults were defined. Those faults contributed in the depression or the lift of different parts of the basin (Minus and plus symbols in respect of figure 6). 

It should be mentioned that the interpretation of the isodepth map was restricted in the area where enough borehole data are available, so as for the results to be as accurate as possible. Moreover, this interpretation was based on the assumption that the existence of a fault is most probable in sites where the number of the wells increases and the depth-differentiation becomes steeper.
Finally, a rose diagram of the resulting faults was constructed (Figure 7). According to this analysis, two main fault directions predominate in the region, namely NNW-SSE and ENE-WSW.

	[image: image7.png]Stratigraphy

Plio_Quaternary

Neogene





	

	[image: image8.png]Histogram
Residuals o cross-valdation

Statistical Summary

g6 apm ™ 1554

@

N ==





	[image: image9.png]Corelation Coefficient

: i £

% . 2

: - :
E3 Ot rved values. oo




	


Figure 5: 3D presentation of the upper Plio-Quaternary surface (transparent surface) and the boundary surface between Plio-Quaternary and Neogene deposits (lower surface). The spatial position of all boreholes is also shown. In the right side are shown the diagrams that resulted from the cross-validation procedure.

5. Conclusions

In the present work, the litho-stratigraphic study of the area is combined with morphotectonic conclusions of previous study (Peterek & Schwarze, 2004); and as a result an enhanced concept regarding the tectono-stratigraphical structure of Tympaki basin arises.
The study of the lithostratigraphy, as well as the upper surface of Neogene formation provide strong evidence of faults that are not observed in the surface because the area is covered by recent fluvio-lacustrine deposits. 

The analysis of the available data was conducted using an isodepth map of the top surface of the Neogene that resulted from the 3D stratigraphical model, which was constructed using a 3D geological modelling software (RockWorks15, RockWare, Inc.). 

Two main fault directions were identified through the analysis. The first one has a general direction NNW-SSE, similar to the strike direction that previous studies have proposed (FAO, 1972; Peterek & Schwarze, 2004; Paritsis, 2005). The second one has a general direction from ENE to WSW direction, and it follows the strike direction of the faults that bound Tympaki basin in the Northern and Southern limit. 
[image: image10.png]Easting —
s0000 sr2000 s4000 st6.000 sra000
—— f f f 60

3,886,000

=

Northing

3880000
880,000

566000 568,000
S T

L
3886.000
=)

-20

-40

Northing

-80

-100

-120

L
3880000

-140

-160

-180

-200

L | | | L L
566,000 568,000 570,000 572000 574,000 576,000 578,000
Easting -220





Figure 6: Isodepth map of the upper surface of Neogene deposits in Tympaki basin. Legend shows absolute elevation in meters. The location of thirty nine (39) boreholes is also shown. Dashed lines depict inferred faults. Plus and minus symbols refer to horsts and grabens respectively.

[image: image11.png]ol

¢l




Figure 7: Rose diagram (percent of total population) of the inferred faults resulted from the interpretation of the isodepth map (Figure 5). Bin size 10o. 

In other words, the drilling data lead to conclusions that are consistent with previous studies regarding the NNW-SSE strike direction; additionally, they underline the existence of a second one pronounced fault with strike direction ENE-WSW that has not been taken into account in previous underground water modelling studies. The latter direction could favour seawater intrusions because exhibit a more or less perpendicular to the coastline position. 
The possibility of parallel activation of the two fault sets should make us to consider a more complicated fault-block structure of Tympaki basin that is characterized by a horst-graben structure and the simultaneous activity of crossing normal faults.

In the area where the number of the boreholes is adequate, the isodepth map defines accurately the horst and graben structure (Figure 6). However, where data were sparse the results are questionable and they should be checked by further drilling or geophysical investigation. According to the results, the top surface of the Neogene deposits has been raised in the central area of the basin, representing a horst structure. Such a conclusion should be checked with geophysical investigation up to depth of about 60 to 80m. Nevertheless, based on the available data it seems quite reasonable to reconsider the tectonostratigraphic structure of Tympaki basin and to investigate the seawater intrusion using advanced research techniques.  
6. Acknowledgments

This study is part of the work that Technical University of Crete undertakes within the Work Package 1 of the BLACK SEA ERA.NET project, focusing in pollution from sea water intrusion at coastal areas (Tympaki basin).

7. References
Bonneau, M. 1985. Geological map of Greece, 1:50.000, Melambes sheet, I.G.M.E., Athens.
Bonneau, M. 1984. Correlation of the Hellenide nappes in the south-east Aegean and their tectonic reconstruction. In: DIXON, J. E. & ROBERTSON, A. H. F. (eds) The Geological Evolution of the Eastern Mediterranean. Geological Society, London, Special Publications, 17, 517-527.

Chatzaras, V., Xypolias, P., Doutsos, T. 2006. Exhumation of high-pressure rocks under continuous compression: a working hypothesis for the southern Hellenides (central Crete, Greece). Geological Magazine, 143, 859-877.

Fallara, F., Legault, M., Rabeau, O. 2006. 3-D integrated geological modeling in the Abitidi Subprovince (Quebec, Canada): Techniques and Applications. Exploration and Mining Geology, 15, 1-2, 27-41.

FAO 1972. Study of the water resources and their exploitation for irrigation in eastern Crete – Greece. Drillings and pumping tests in Messara AGL:SF/GRE 17/31 tech. rep. 26, UNDP, Iraklio.

FAO 1969. Greece: geophysical survey performed in Messara plain (Eastern Crete). Final report. Fondazione Ing. C. M. Lerici Del Politecnico, Milan, Italy.
Jolivet, L., Goffe, B., Monie, P., Truffert-Luxey, C., Patriat, M., Bonneau, M. 1996. Miocene detachment in Crete and exhumation P–T–t paths of high-pressure metamorphic rocks. Tectonics, 15, 1129–1153
Kaufmann, O., Martin, T. 2008. 3D geological modeling from boreholes, cross-sections and geological maps, application over former natural gas storages in coal mines. Computer & Geosciences, 34, 278-290.
Kilias, A. A., Tranos, M. N., Oroxco, M., Alonsochaves, F. M and Soto, J. I. 2002. Extensional collapse of the Hellenides: a review. Revista de la Sociedad Geological de Espania, 15, 129-139.
Kilias, A., Fassoulas, C., Mountrakis, D. 1994. Tertiary extension of continental crust and uplift of Psiloritis metamorphic core complex in the central part of the Hellenic Arc (Crete Greece). Geol. Rundschau, 83, 417–430.
Klein, T., Craddock, J. P., Zulauf, G. 2012. Constraints on the geodynamical evolution of Crete: insights from illite crystallinity, Raman spectroscopy and calcite twinning above and below the ‘Cretan detachment’. International Journal of Earth Sciences, 102, 139-182.

Kokinou, E., Alves, T., Kamberis E. 2012. Structural decoupling in a convergent forearc setting (southern Crete, Eastern Mediterranean). Geological Society of America Bulletin, 124, 1352-1364.

Lister, G.S., Banga, G., Feenstra, A. 1984. Metamorphic core complexes of cordilleran type in the Cyclades, Aegean Sea, Greece. Geology, 12, 221–225.

Meulenkamp, J. E. 1979. Field Guide to the Neogene of Crete. In: Symeonidis, N., Papanikolaou, D., Dermitzakis M. (Eds.), Field Guide to the Neogene of Crete. Publ. Dept. Geol. Paleont. Univ. Athens, Series A, 32, 1–32.
Mountrakis, D., Tranos, M., Papazachos, C., Thomaidou, E., Karagianni, e. & Vamvakaris, D. 2006. Neotectonic and seismological data concerning major active faults, and the stress regimes of Northern Greece. In: ROBERTSON, m. n. F. & MOUNTRAKIS, D. (eds) Tectonic Development of the Eastern Mediterranean Region. Geological Society, London, Special Publications, 260, 649-670.
McClusky, S., Balassania, S.,  Barka, A., Demir, C., Ergintav, S., Georgiev, I., Gurkan, O., Hamburger, M., Hurst, K., Kahle, H., Kastens, L., Kekelidze, G., King, R. W., Kotzev, V., Lenk, O., Mahmoud, S., Mishin, A., Nadariya, M., Ouzounis, A., Paradissis, D., Peter, Y., Prilepin, M., Reilinger, R., Sanli, I., Seeger, H., Taeleb, A., Toksoz, M. N., and Veis, G. 2000. GPS constraints on plate motions and deformation in the Eastern Mediterranean: Implications for plate dynamics. J. Geophys. Res., 105, 5695–5719.
National Casastre & Mapping Agency S.A.. DEM with grid size 5m x 5m.

Oral, M. B., Reilinger, R. E., Toksoz, M. N., Kong, R. W., Barka, A. A., Kinik, I. & Kenk, O. 1995. Global positioning system offers evidence of plate motions in eastern Mediterranean. EOS Transactions. American Geophysical Union, 76 (9), 9-11.

Papazachos, C.B. 1993. Seismological and GPS evidence for the Aegean-Anatolia interaction. Geophysical Research Letters, 26, 17, 2653-2656.

Paritsis, S.N. 2005. Simulation of seawater intrusion into the Tymbaki aquifer, South Central Crete, Greece. Report within MEDIS project, Study implemented on behalf of the Department of Management of Water Resources of the Region of Crete. Heraklion, Crete, Greece.

Peterek, A., Schwarze, J. 2004. Architecture and Late Pliocene to recent evolution of outer-arc basins of the Hellenic subduction zone (south-central Crete, Greece). Journal of Geodynamics, 38, 19-55.
Reilinger, R., et al. 1997. Global Positioning System measurements of present-day crustal movements in the Arabia–Africa–Eurasia plate collision zone. J. Geophys. Res., 102, 9983–9999.
Ring, U., Glodny, J., Will, T., Thomson, S. 2010. The Hellenic Subduction System: High-Pressure Metamorphism, Exhumation, Normal Faulting, and Large-Scale Extension. DOI:10.1146/annurev.earth.050708.170910 pp.45-76.

Seidel, E., Kreuzer, H., Harre, W. 1982. A Late Oligocene/Early Miocene High Pressure Belt in the External Hellenides. Geol. Jb., E 23, 165–206.
Tortorici, L., Caputo, R., Monaco, C. 2010. Late Neogene to Quaternary contractional structures in Crete (Greece). Tectonophysics, 483, 203-213.

Vidakis, M., Jonkers, A., Meulenkamp, J. E. 1994. Geological map of Greece, 1:50.000, Epano Archanae sheet, I.G.M.E., Athens.


